Pagnon, Andrew C.

acpagnon@cpp.edu

schematic entry & simulation using cadence orcad, pspice and excel

ece2200 activity

Contents

1.	Launching PSpice & creating a new project	3

I.	Launch Cadence:	3

i.	Open Design Entry CIS	3

II.	Create a new project	3

ii.	New Project	3

iii.	Add Project path	3

iv.	Enter project name	3

v.	Select Analog or Mixed A/D	3

vi.	Click <OK> on the “New Project’ pop-up window below	4

vii.	Create blank project	4

2.	Create a Schematic	5

III.	Add Libraries	5

viii.	Select Place part icon	5

ix.	Select the add Library Icon	5

x.	Browse for Libraries	6

xi.	Select all Libraries	6

xii.	Add Libraries	6

IV.	Placing parts	6

xiii.	Adding a resistor part and rotating it	6

xiv.	Adding an NMOS transistor	7

xv.	Adding a ground:	8

xvi.	Create the intermediate schematic shown below:	9

V.	Connecting parts with wires	10

xvii.	Adding wires:	10

xviii.	Drawing wires (shown below)	10

xix.	Add wires to connect the following circuit	11

xx.	Naming wires:	11

VI.	Modifying Properties	13

xxi.	Select and change Resistor Component value	13

xxii.	Select and change Voltage Source value	13

xxiii.	Sampling of notations that PSPICE recognizes	13

VII.	Save schematic	13

3.	DC Bias Simulation	14

VIII.	Creating a Variable	14

xxiv.	Enter component variable value	14

xxv.	Add Variable definition part PARAM	14

xxvi.	Exit create variable command	14

xxvii.	Enter Variable property name and value in PARAM	14

IX.	Placing Current and Voltage Probes	16

xxviii.	Place voltage probes to plot voltages or current probes for currents.	16

X.	DC sweep with a Nested Parametric sweep simulation	17

xxix.	Create a simulation profile	17

xxx.	Edit Primary Sweep Parameters	18

xxxi.	Edit Parametric Sweep Parameters	19

xxxii.	Simulate	20

xxxiii.	Netlist Generation	21

xxxiv.	Errors – Session Log	21

xxxv.	Parametric Sweep options	22

xxxvi.	Add simulation traces	22

xxxvii.	Simulation waveform window	23

xxxviii.	Export simulation data to Excel	23

4.	Excel Data Analysis – Plot data	24

XI.	Plot excel data	24

xxxix.	Format Excel data	24

xl.	Plot Data	26

xli.	Analyze excel data (Load Line analysis)	28

xlii.	Design Bias circuit parameters	28

5.	Bias Point Simulation	29

XII.	Replace the parametric sweep Voltage source with 2 resistor bias circuit	29

xliii.	Modify schematic	29

XIII.	Run Bias Point Simulation	30

xliv.	Edit Simulation profile	30

xlv.	Run Simulation	30

6.	Time Domain (Transient) Simulation	32

XIV.	Replace 2 resistor bias circuit with AC Voltage Source VSIN/SOURCE	32

xlvi.	Modify Schematics	32

XV.	Run Time Domain (Transient) Simulation	33

xlvii.	Edit Simulation profile	33

xlviii.	Run Simulation	34

7.	Quiz 7 – May 23th 2018	36

How to use PSPICE – Schematic design, simulation, analysis

# Launching PSpice & creating a new project

## Launch Cadence:

### Open Design Entry CIS

Start→ type ‘cis’ → click on ‘Design Entry CIS’ or ‘Capture CIS’ → click OK on selection below

## Create a new project

### New Project

On the top toolbar select:  File->New->Project

### Add Project path

In your student account on the H: drive
in ‘Location’ click on ‘Browse’

### Enter project name

For example ece2200l_m1

### Select Analog or Mixed A/D

So you will be able to simulate

### Click <OK> on the “New Project’ pop-up window below

### Create blank project

In the pop-up menu select: Create blank project then click <OK>

# Create a Schematic

## Add Libraries

### Select Place part icon

Place part icon

### Select the add Library Icon

Add Library icon

### Browse for Libraries

In the Browse File pop-up menu select ‘Look in’ 
Local Disk (C:) Cadence/SPB_17.2/tools/capture/library/pspice

### Select all Libraries

Click on the blank part of the window and hit <ctrl a> to select all the libraries (see below)

### Add Libraries

Select Open to add the selected libraries (see below)

click here and select <ctrl a>

Add path

## Placing parts

### Adding a resistor part and rotating it

#### Search for Part

In the Place part pop-up menu enter the name of the part you wish to add (Here we have entered R)

IMPORTANT:  You can find the part R in several libraries. Ensure all libraries are selected

Select the part R from the library Analog.  (Notation:  R/ANALOG)

#### Add the part

Hit <enter> to add the part

If the part was not in the design cache you will see a pop-up menu.

Hit <yes> If you should get this menu.

Move your cursor on the schematic to where you want to place the part.

Click r to rotate it

Click your mouse on the schematic screen to place the resistor

You will continue to be able to place parts that are of the type that you selected.  Hit <esc> to end placing parts.

#### Bindkeys for rotating or flipping

If you wish to rotate or flip a part as you place it use the following bindkeys

##### Rotate		r

##### Horizontal flip	h

##### Vertical flip	v

Select the part R from the ANALOG library

### Adding an NMOS transistor

#### Search for NMOS transistor

Click hotkey p to place part

Select part MbreakN from the BREAKOUT library

click <OK> to close the window

#### Place the part

left click with your mouse to position the NMOS in your schematic window

click <esc> to stop placing the part

#### Common Editing Commands

p	Place part               w	Create wire

^ c	Copy        ^ c 	cut            ^ v	paste

r	rotate

v	flip vertically

h	flip horizontally

<delete>	delete

Parts can be repositioned by dragging them

### Adding a ground:

#### Search for Ground/0

Select bindkey G OR click ground icon

Select:  0/CAPSYM

#### Place Ground

Hit <OK> to start placing the ground

Hit : <esc>to end the place ground part command

### Create the intermediate schematic shown below:

#### Add 2 resistors R from the library ANALOG (rotated vertical)

#### Add 2 DC voltage sources Vsrc from the library SOURCE.

#### Add 1 NMOS transistor MbreakN from the library BREAKOUT

#### Note parts can be repositioned by dragging them.

After placing the parts you will need to connect them with wires (also called nets).

## Connecting parts with wires

Place wire icon

Adding wires:

#### Select wire tool

Select the Place wire icon (toolbar on rhs) OR use bindkey w

### Drawing wires (shown below)

#### Start wire

Click your mouse at the starting position of your wire

#### Bend wire

Move (do not drag) your mouse to where you want a bend. 

Click your mouse. (You may repeat this step for multiple bends)

#### End wire

Move (do not drag) your mouse to the end location click again.

#### Exit wire command

Hit the <esc> key to exit the place wire command.

Click mouse (starting point)

Move mouse

Click mouse (bend)

Move mouse

Click mouse (bend)

Move mouse

Click mouse (end point)

Hit <esc> to end command

### Add wires to connect the following circuit

### Naming wires:

It is much easier to maintain a design if you name the wires (nets) on your schematic.  
Try to create schematics that are readable and less cluttered with wires

Below are two equivalent schematics. Which is more readable/maintainable?

IMPORTANT ideas - If wires have the same name (net alias) they are viewed as being connected - To simulate a PSpice schematic there needs to be a net named 0 (zero)

#### Name a wire (net):

##### Create wire Name

Selecting bindkey n OR

Selecting the place net alias icon (right hand side toolbar) OR

Selecting the Place->net alias (top toolbar)

In the resulting pop-up menu enter the name (Vin for this example)

##### Place wire name

Click your mouse on the net that you wish to name.  (The input of the amplifier.)

Click your mouse again on the wire at the top of one of the two voltage sources

##### End name command

Hit <esc> to end the place net alias command

Note:  do NOT use the place text icon.  PSpice text offers no functionality.  It is, however, helpful for annotating schematics with comments.

#### Name the wires on your schematic as follows

Place net alias icon

Place text icon

## Modifying Properties

### Select and change Resistor Component value

Double click the value of resistors (1K) R1 and R2.

Change them to 470 and 150

Click <OK> to close the window.

### Select and change Voltage Source value

Double click the DC value of the voltage source (source V2) (DC=)

Change it to be 15V, click <OK> to close the window.

Right Click on M1 a select “Edit Spice Model”

An “AMS Model Editor” window should open. May be minimized.

Type “Kp=9m Vto=0.7V” after “.model Mbreakn NMOS”

Click on “Save Library” icon

### Sampling of notations that PSPICE recognizes

|  | PSpice Notation |  |  |  |  | PSpice Notation |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 103 | K | E3 |  |  | 10-3 | m | E-3 |
| 106 | Meg (not M) | E6 |  |  | 10-6 | u | E-6 |
| 109 | G | E9 |  |  | 10-9 | n | E-9 |
|  |  |  |  |  | 10-12 | p (pico) | E-12 |
|  |  |  |  |  | 10-15 | f (femto) | E-15 |

## Save schematic

From the main menu: File->Save

Your schematic is now complete!

# DC Bias Simulation

## Creating a Variable

### Enter component variable value

Double click on the DC= of the voltage source V1

Change it to the equation {ving*1V} (curly brackets are needed)

ving is a variable.

### Add Variable definition part PARAM

Hit bindkey p and add the part PARAM from the library special.

This part defines variables

### Exit create variable command

Click on <OK> to close the window and place the part

### Enter Variable property name and value in PARAM

On the schematic, double click on the word PARAMETERS.

You will see the window below.

Click on New Column (or New Row or New Property)  - It is different for different versions

If prompted answer yes

In the Add New Column window type the variable name

Give it a default value (as shown below)

Click OK to close the Add New Column window

Close window

Close the Property Editor Window (top right)
Now you are ready to simulate

## Placing Current and Voltage Probes

### Place voltage probes to plot voltages or current probes for currents.

Automatically plots results when a simulation is complete

Place a voltage probe at the output of the amplifier as shown below

Place current probe to plot the collector current.

Note current probes must be placed on the pins of devices (the very edge – look for the red/blue transition)

## DC sweep with a Nested Parametric sweep simulation

### Create a simulation profile

Select Pspice->New Simulation Profile from the top toolbar

Enter a name (like ece2200l_sim_m1)

Click on create to close the window

A Simulation Settings window will open (may be minimized)

### Edit Primary Sweep Parameters

Select the analysis tab (it should be the default)

Select DC sweep analysis

Specify that you wish to sweep a voltage source

Specify the voltage source that you wish to sweep (V2)
this one is connected to the output of the amplifier

Specify the Start: 0, stop: 25 voltages as well as the voltage increment: 1

Note: Spice understands m to be 10-3

### Edit Parametric Sweep Parameters

Select Parametric Sweep (ensure it is checked)

Select Global Parameter

Specify the parameter you wish to sweep (ving)

Specify the range for ving (start: 1.5,  stop:  6, Increment 0.75)

Click on <OK> to close the window

### Simulate

Click on the run Pspice icon at the top of the schematic

Run Pspice icon

### Netlist Generation

Pspice will generate a netlist of the schematic

Simulation window minimized

### Errors – Session Log

If your simulation fails check for errors:

from top toolbar:  Window->Session Log

If netlist runs successfully a simulation window will open (may be minimized as above)

### Parametric Sweep options

Click on <OK> to run all seven parametric sweep simulations

### Add simulation traces

Open the simulation window (if minimized) and right click on waveform screen

Select “add traces” and add the 4 waveforms I(M1:d) VG(M1) VD(M1) VS(M1)

### Simulation waveform window

The simulation waveforms will be displayed as below

### Export simulation data to Excel

Select File -> Export -> comma-separated values (.csv file) and click ok

Provide a file name (like ece299_sim_m1) and save in your H drive account

# Excel Data Analysis – Plot data

## Plot excel data

### Format Excel data

Open excel csv file

Select all cells (click on top right corner)

Right click on cell area and select “format cells”

Double click to format cell width

Click to select all cells

Create columns for Id in mA (multiply Id by 1000) and Vds (Vd-Vs) for each of the 7 Vg values

Create columns for Vgs (Vg-Vs), Vdsat for each of the 7 Vg values

You can do this pretty quickly in Excel. For example for Vds (showing only 2 values below instead of 7)

Find the first free column (H in the example below)

Write Vds1 in the first row

In the second row write “=”, click on the first value from the “(V(M1:d))@1” column, write “-”, click on the first value from the “(V(M1:s))@1” column. Type Enter

Double click on the bottom right corner of the cell (cursor should become a cross). This will autofill the column.

Select all the cells filled in this column (including Vds1 on row 1)

Grab the bottom right corner of this selection and drag it right across 6 empty columns to auto fill them

Double Click to autofill

Grab and drag right across 6 empty columns to autofill them

### Plot Data

Create XY scatter plots for Id versus Vds in Excel with these steps:

Select the “Insert” tab and under “Charts” click on “Scatter with straight lines and Markers”

Right click on the new chart area and click on “Select Data”

Click on “Add” under “Legend Entries” and fill in the “Series name” (Id1), “Series X Values” by selecting all the Vds1 values (column H in the example below) and “Series Y Values” by selecting the values in Column B below. Hit OK.

Click “Add” 6 more times to repeat the “Select Data” process for Id2 to Id7

Create XY scatter plots for Id versus Vo (Vd)

### Analyze excel data (Load Line analysis)

Add a load line for Id versus Vo from Vomax (15V) to Vomin (5.26V)

Label 2 free columns as “VLoad” and “ILoad”

In the VLoad column enter the Vomin (5.26V) and Vomax (15V) values

In the Iload column enter the current at Vomin in mA (roughly the highest current with Vg=1.5V, about 20.95mA), and at Vomax (ILoad=0mA)

Add a new series to the Scatter plot  with X axis Vload and Y axis ILoad

### Design Bias circuit parameters

Graphically determine ideal DC bias point operating input and output parameters

Amplifier output voltage VOQ = about 10V

Amplifier output current IDQ= about 10mA

Amplifier input Voltage VINQ = Vg = about 3.75V

# Bias Point Simulation

## Replace the parametric sweep Voltage source with 2 resistor bias circuit

### Modify schematic

Remove “V1 voltage Source” and “Parameter”

Add 2 resistors (112.5K, 37.5K) for a 2-resistor Vg bias circuit to set Vg to about 3.75V

Note the large resistance values to minimize the impact on the NMOS bias current

Ensure M1 parameters are still KP=9m and VTO=0.7V

## Run Bias Point Simulation

### Edit Simulation profile

Analysis type = Bias Point

### Run Simulation

#### Enable Bias Voltage Display

#### Enable Bias Current Display

# Time Domain (Transient) Simulation

## Replace 2 resistor bias circuit with AC Voltage Source VSIN/SOURCE

### Modify Schematics

Replace V2 with a VSIN/SOURCE part

Set VSIN parameters

VOFF = 5V (Offset voltage)

VAMPL = 5V (Peak Voltage)

FREQ = 1K (1KHz sinewave)

## Run Time Domain (Transient) Simulation

### Edit Simulation profile

Analysis type = Time Domain (Transient)

Run To Time = 2ms

Maximum Step Size = 0.1m

### Run Simulation

#### Open simulation window, right click and add traces V(Vin) and V(Vo)

#### Analyze Simulation Results

CutOff

Vo=15V (max)

Vgs<Vtn=0.7V

Linear/Triode

Vgs>Vtn

Vds<Vdsat

Poor gain

Saturation

Vgs>Vtn

Vds>Vdsat

Maximum gain

VOQ

DC Bias Operating Point when VIN=3.75V and VO=10V

Identify the 3 different NMOS transistor mode regions

Identify DC Bias Operating Point

Identify Amplifier Gain VoVin=15-56-1=2

# Quiz 7 – May 23th 2018

Assume:

KN=9mA/V2

VTN=0.7V

Equations (when MOSFET is in Saturation.):

Id=Kn2*Vdsat2

Vdsat=Vgs-VTn

Determine R2 and R3 to bias M1 at midswing Vo

Vomax=15V when M1 is in cutoff and Id=0

Vomin at the onset of saturation when Vds=Vdsat

KVL from 15V supply rail to ground through M1

15V-470Ω*Idmin-Vds-150Ω*Idmin=0

15V-620Ω*Idmin-Vdsatmin=0

Id=Kn2*Vdsat2

15V-620Ω*9mAV2*Vdsatmin2-Vdsatmin=0

Vdsatmin = -2.5048V or 2.1464V.

Since M1 is on at Vomin, then Vdsatmin = 2.1464V>0V

Idmin=Kn2*vdsatmin2

Idmin=4.5mAV2*(2.1464V)2=20.7316mA=Idmin

Vomin=150Ω*Idmin+Vdsatmin=150Ω*20.7316mA+2.1464V=5.2561V=Vomin

Voqmidswing Vo=Vomax+Vomin2=15V+5.2561V2=10.128V=Voq

Idq=15V-Voq470Ω=15V-10.128V470Ω=10.366mA=Idq

Idq=Kn*Vdsatq2=10.366mA=4.5mAV2*Vdsatq2

Vdsatq=1.5177V=Vgsq-VTn=Vgsq-0.7V

Vgsq=2.2177V

Vgq=Vgsq+150Ω*Idq=2.2177V+150Ω*10.366mA=3.7726V=Vgq

Vgq=15V*R3R2+R3

R2=R3*15V-VgqVgq

If R3=37.726KΩ, then R2=37.726KΩ*15V-3.78V3,78V=112.274KΩ=R2 and R2+R3=150KΩ>>R1+R4